# Mathematical Documentation: Cathode Ray Tube as a Physics-Based Generative Image Model

## 1. Problem Definition

A Cathode Ray Tube (CRT) can be mathematically modeled as a physics-based image generation system. In a CRT, an electron beam is generated, accelerated, deflected across a phosphor-coated screen, and intensity-modulated to produce visible light. Therefore, the final image is not created directly as a matrix of pixels; instead, it is generated through a continuous physical process involving electron motion, scanning signals, beam intensity, and phosphor response.

The goal is to define a mathematical function that maps input electrical signals and physical CRT parameters to an output image:

I=CRTVxt,Vyt,Bt,θ

where:

| Symbol | Meaning |
| --- | --- |
| I | Generated image |
| Vxt | Horizontal deflection signal |
| Vyt | Vertical deflection signal |
| Bt | Beam brightness or intensity modulation signal |
| θ | Physical CRT parameters |
| t | Continuous time |

In machine learning terms, this can be interpreted as a physics-constrained generator. Instead of allowing a neural network to generate pixels freely, the model generates images through physically meaningful intermediate variables: beam position, scan trajectory, spot shape, phosphor response, and noise.

This idea is related to generative modeling because GANs define a generator G that maps latent variables to images, while a CRT-inspired model maps voltage signals and physical parameters to images. GANs were introduced as a two-player adversarial framework in which a generator learns to produce samples that confuse a discriminator (Goodfellow et al., 2014).

Thus, a CRT generator may be written as:

GCRTz,θ→I

where z is a latent vector controlling scan distortion, beam noise, brightness variation, blur, screen curvature, and other visual effects.

## 2. Physical Components of the CRT

A CRT image formation system consists of the following main components:

### 2.1 Electron Gun

The electron gun emits electrons inside a vacuum tube. In a simplified CRT model, the electron gun is treated as a source of charged particles. The emitted electrons are accelerated toward the screen by an electric potential difference.

### 2.2 Cathode and Thermionic Emission

The cathode is heated so that electrons are emitted from its surface. For the first version of the mathematical model, thermionic emission can be simplified by assuming that a controllable electron beam current Bt is available.

### 2.3 Accelerating Anode Voltage

The accelerating anode creates a potential difference Va, which gives kinetic energy to the electrons. This determines the forward velocity of the electron beam.

### 2.4 Deflection System

The beam must be steered across the screen. Two types of deflection can be modeled:

| Deflection type | Typical use | Mathematical basis |
| --- | --- | --- |
| Electrostatic deflection | Oscilloscopes, simplified CRT models | Electric force F=qE |
| Magnetic deflection | Television CRTs and monitors | Lorentz force F=qv×B |

Electrostatic deflection is easier for the first mathematical model because it gives direct equations between voltage and screen displacement. Magnetic deflection is closer to many television CRTs, but it introduces field geometry and coil dynamics. CRTs use electron guns, deflection systems, and phosphor screens to form images by steering electron beams onto the screen.

### 2.5 Phosphor Screen

The screen is coated with phosphor material. When electrons hit the phosphor layer, visible light is emitted. The light response may persist for a short time after the beam has moved away, producing phosphor persistence.

### 2.6 Raster Scanning

In television-like CRTs, the beam scans the screen line by line. The horizontal scan moves quickly from left to right, while the vertical scan moves slowly from top to bottom. The Tektronix oscilloscope primer describes the CRT beam sweeping horizontally across a phosphor-coated screen, creating a glowing trace where the beam strikes the phosphor.

### 2.7 Brightness Modulation

The image brightness is controlled by modulating the beam intensity over time. In a grayscale CRT model, the beam intensity Bt controls the brightness at the beam location. In a color CRT model, three intensity signals may be used:

BRt, BGt, BBt

for red, green, and blue phosphor channels.

## 3. Electron Acceleration Model

The first mathematical step is to describe how electrons are accelerated inside the CRT.

Let:

| Symbol | Meaning | Unit |
| --- | --- | --- |
| m | Electron mass | kg |
| e | Magnitude of electron charge | C |
| Va | Accelerating voltage | V |
| v | Electron velocity after acceleration | m/s |

Assuming the electron starts approximately from rest and relativistic effects are ignored, the electrical potential energy is converted into kinetic energy:

eVa=12mv2

Solving for velocity:

v=2eVam

This equation shows that increasing the accelerating voltage increases the electron velocity. A faster electron beam reaches the screen more quickly but becomes harder to deflect.

### 3.1 Non-Relativistic Assumption

The above equation is valid when the electron velocity is much smaller than the speed of light:

v≪c

where c is the speed of light.

If the accelerating voltage is very high, relativistic correction may be required. The relativistic kinetic energy equation is:

eVa=γ-1mc2

where:

γ=11-v2c2

For a first implementation, the non-relativistic equation is sufficient unless the CRT model is intended to simulate high-voltage electron dynamics very accurately.

## 4. Electrostatic Beam Deflection Model

For implementation clarity, we first model electrostatic deflection. This model is suitable for explaining the basic mathematics of CRT beam motion.

Assume the electron beam travels mainly in the z-direction toward the screen. The horizontal and vertical deflection plates create electric fields that push the beam in the x and y directions.

### 4.1 Electric Field Between Deflection Plates

For two parallel plates separated by distance d, with deflection voltage Vd, the electric field is approximated as:

E=Vdd

where:

| Symbol | Meaning | Unit |
| --- | --- | --- |
| E | Electric field magnitude | V/m |
| Vd | Deflection voltage | V |
| d | Plate separation | m |

### 4.2 Force on the Electron

The electric force on a charged particle is:

F=qE

For an electron, q=-e. Using magnitude:

F=eE=eVdd

### 4.3 Transverse Acceleration

Using Newton’s second law:

F=ma

so the transverse acceleration is:

a=Fm=eVdmd

This means that the deflection acceleration is proportional to the deflection voltage and inversely proportional to the plate separation.

## 5. Vertical Deflection Derivation

Assume the electron enters the deflection region with forward velocity v. Let:

| Symbol | Meaning |
| --- | --- |
| L | Length of deflection plates |
| D | Drift distance from end of plates to screen |
| v | Forward electron velocity |
| ay | Vertical acceleration |
| y | Final vertical displacement on screen |

The time spent between the plates is:

tp=Lv

The vertical displacement inside the plates is:

y1=12aytp2

Substituting tp=L/v:

y1=12ayLv2

The vertical velocity gained while inside the plates is:

vy=aytp=ayLv

After leaving the plates, the electron travels a drift distance D. The drift time is:

tD=Dv

The additional vertical displacement during drift is:

y2=vytD

Substituting:

y2=ayLvDv y2=ayLDv2

Therefore, the total vertical deflection is:

y=y1+y2 y=12ayL2v2+ayLDv2 y=ayLv2D+L2

Substitute:

ay=eVymdy

Therefore:

y=eVyLymdyv2Dy+Ly2

Now substitute:

v2=2eVam

Then:

y=VyLy2dyVaDy+Ly2

This is one of the most important equations for implementation:

yt=SyVyt

where:

Sy=Ly2dyVaDy+Ly2

Here, Sy is the vertical deflection sensitivity.

## 6. Horizontal Deflection Derivation

Similarly, for horizontal deflection:

xt=SxVxt

where:

Sx=Lx2dxVaDx+Lx2

Thus:

xt=VxtLx2dxVaDx+Lx2

and:

yt=VytLy2dyVaDy+Ly2

These two equations map deflection voltages directly to beam position on the screen.

## 7. Raster Scan Mathematical Model

A raster scan forms an image by moving the electron beam across the screen in repeated horizontal lines.

Let the image have:

| Symbol | Meaning |
| --- | --- |
| W | Image width in pixels |
| H | Image height in pixels |
| Tf | Frame duration |
| ff | Frame rate |
| Th | Horizontal line duration |
| fh | Horizontal scan frequency |

The frame duration is:

Tf=1ff

The horizontal line duration is:

Th=TfH

The horizontal scan can be modeled using a normalized sawtooth function:

sht=2t mod ThTh-1

so:

xt=Xmaxsht

where Xmax is the maximum horizontal screen coordinate.

The vertical scan is slower:

svt=2t mod TfTf-1

so:

yt=Ymaxsvt

where Ymax is the maximum vertical screen coordinate.

Therefore, the scan trajectory is:

xt,yt=Xmaxsht,Ymaxsvt

This is the simplified raster trajectory. In a more realistic CRT model, retrace intervals should be included:

xt=active scan,0≤th<Tactiveretrace,Tactive≤th<Th

where th=t mod Th.

## 8. Intensity Formation Model

The beam intensity signal Bt controls the brightness of the point where the beam hits the phosphor screen.

For an ideal point beam:

Ixt,yt=Bt

However, a real electron beam has finite spot size. Therefore, instead of illuminating a single point, the beam creates a small light distribution around the impact point.

Let Kx,y be the beam spot function or point spread function. A common simplified model is a 2D Gaussian:

Kx,y=12πσ2exp-x2+y22σ2

where σ controls beam blur.

The image intensity can then be modeled as:

Ix,y=∫BtKx-xt,y-ytdt

For digital implementation, this becomes:

Ii,j=n=1NBnKi-xn,j-yn

where:

| Symbol | Meaning |
| --- | --- |
| Ii,j | Pixel intensity at row i, column j |
| Bn | Beam brightness at time step n |
| xn,yn | Beam position at time step n |
| K | Beam spot kernel |
| N | Number of sampled time steps |

## 9. Phosphor Persistence Model

Phosphor persistence means that brightness does not disappear instantly after the electron beam leaves a point. A simple temporal decay model is:

Pt=P0e-t/τ

where:

| Symbol | Meaning |
| --- | --- |
| Pt | Phosphor brightness after time t |
| P0 | Initial brightness |
| τ | Phosphor decay constant |

If the beam deposits energy at time tn, then the contribution at a later time t is:

Bne-t-tn/τ

for t≥tn.

Thus, the full image intensity at time t can be modeled as:

Ix,y,t=n:tn≤t​Bne-t-tn/τKx-xn,y-yn

This equation is important because it allows the model to generate CRT-like visual effects such as glow, persistence, afterimage, ghosting, and motion trails. Phosphor persistence is a known CRT display property, and phosphor type affects brightness, color, and persistence duration.

## 10. Complete CRT Image Formation Equation

Combining the previous components, a simplified CRT image formation model is:

Ix,y,t=n:tn≤t​Btne-t-tn/τKx-xtn,y-ytn

where:

xtn=SxVxtn ytn=SyVytn

and:

Sx=Lx2dxVaDx+Lx2 Sy=Ly2dyVaDy+Ly2

Therefore:

I=CRTVxt,Vyt,Bt,Va,Lx,Ly,dx,dy,Dx,Dy,σ,τ

This is the core mathematical object that the machine learning team can implement.

## 11. Connection to Generative Machine Learning

A traditional GAN generator maps a latent vector z to an image:

Gz→I

A CRT-inspired generator maps latent variables and physical parameters to voltage signals and then to an image:

z→{Vxt,Vyt,Bt,θ}→I

Thus:

GCRTz=CRTVxt;z,Vyt;z,Bt;z,θz

The latent vector z may control:

| Latent variable effect | Mathematical role |
| --- | --- |
| Image content | Controls Bt |
| Horizontal distortion | Perturbs Vxt |
| Vertical distortion | Perturbs Vyt |
| Brightness variation | Scales Bt |
| Blur | Controls σ |
| Persistence | Controls τ |
| Noise | Adds stochastic perturbation |
| Screen curvature | Warps x,y |
| Scanline artifacts | Modifies raster sampling |

GANs are suitable when the goal is high-quality image generation through adversarial learning (Goodfellow et al., 2014). VAEs are suitable when the goal is a structured latent representation and probabilistic generation (Kingma and Welling, 2013). Diffusion models are suitable when generation is framed as iterative denoising from noise (Ho, Jain and Abbeel, 2020). Physics-informed neural networks are relevant when physical equations or constraints must be respected during learning (Raissi, Perdikaris and Karniadakis, 2019).

## 12. Parameterization for Machine Learning

The model can be separated into fixed physical parameters, learnable physical parameters, input signals, and output tensors.

### 12.1 Fixed Physical Parameters

| Parameter | Meaning |
| --- | --- |
| e | Electron charge |
| m | Electron mass |
| c | Speed of light |
| W,H | Image width and height |
| ff | Frame rate |
| fh | Horizontal scan frequency |

### 12.2 Learnable or Tunable Parameters

| Parameter | Meaning |
| --- | --- |
| Va | Effective accelerating voltage |
| σ | Beam spot width |
| τ | Phosphor decay constant |
| Sx,Sy | Deflection sensitivities |
| ηx,ηy | Scan noise |
| γ | Brightness nonlinearity |
| κ | Screen curvature coefficient |

### 12.3 Input Signals

| Signal | Meaning |
| --- | --- |
| Vxt | Horizontal scan signal |
| Vyt | Vertical scan signal |
| Bt | Brightness signal |
| z | Latent vector |

### 12.4 Output

For grayscale:

I∈RH×W

For RGB:

I∈RH×W×3

## 13. Discrete Implementation Model

For implementation, time is sampled:

tn=nΔt

where:

n=0,1,2,…,N-1

The beam position becomes:

xn=xtn, yn=ytn

The brightness becomes:

Bn=Btn

The discrete image is:

Ii,j=n=1NBnKi-yn,j-xn

If phosphor persistence is included:

Ii,j,k=n:tn≤tk​Bne-tk-tn/τKi-yn,j-xn

where k is the frame index.

## 14. Possible Training Objectives

The CRT generator can be trained using a combination of reconstruction, perceptual, adversarial, and physics-based losses.

### 14.1 Pixel Reconstruction Loss

Lpixel=∥Ireal-Igen∥1

or:

Lpixel=∥Ireal-Igen∥22

### 14.2 Adversarial Loss

For GAN-based training:

minGmaxDEx∼pdatalogDx+Ez∼pzlog1-DGCRTz

This follows the general adversarial learning idea introduced by Goodfellow et al. (2014).

### 14.3 Physics-Consistency Loss

To ensure physically meaningful beam motion:

Lphysics=xt-SxVxt2+yt-SyVyt2

### 14.4 Temporal Smoothness Loss

To prevent unrealistic sudden beam jumps:

Lsmooth=n​xn+1-xn2+yn+1-yn2

### 14.5 Signal Regularization Loss

To keep voltage signals within realistic ranges:

Lsignal=n​max0,Vxtn-Vx,max+max0,Vytn-Vy,max

### 14.6 Total Loss

A combined objective can be:

Ltotal=λ1Lpixel+λ2Ladv+λ3Lphysics+λ4Lsmooth+λ5Lsignal

where λ1,…,λ5 are weighting coefficients.

## 15. Assumptions and Simplifications

The first version of the model can use the following assumptions:

| Assumption | Explanation |
| --- | --- |
| Non-relativistic electrons | Use v=2eVa/m |
| Uniform electric field | Assume E=Vd/d |
| Constant forward velocity | Ignore small changes in z-velocity during deflection |
| Gaussian beam spot | Use a simple point spread function |
| Linear phosphor response | Assume brightness proportional to beam intensity |
| Simple exponential decay | Model persistence with e-t/τ |
| No space-charge effects | Ignore electron-electron repulsion |
| No detailed lens focusing | Use σ as effective blur |
| No color mask initially | Start with grayscale CRT |
| No interlacing initially | Use progressive raster scan |

Advanced versions may include:

- Magnetic deflection.

- Relativistic correction.

- Electron lens focusing.

- Beam spread.

- Phosphor nonlinearity.

- Color phosphor triads.

- Shadow mask or aperture grille effects.

- Screen curvature.

- Blooming.

- Flicker.

- Scanline artifacts.

- Interlaced scanning.

## 16. Implementation-Oriented Algorithm

### Step-by-step CRT generator

- Define image size H×W.

- Define time samples tn.

- Generate or learn horizontal voltage signal Vxtn.

- Generate or learn vertical voltage signal Vytn.

- Generate or learn brightness signal Btn.

- Compute deflection sensitivities Sx and Sy.

- Compute beam positions:

- xn=SxVxtn yn=SyVytn

- Convert continuous coordinates xn,yn into image coordinates.

- Deposit brightness using kernel K.

- Apply phosphor persistence if temporal modeling is required.

- Apply optional distortions, noise, curvature, and brightness nonlinearity.

- Output image tensor I.

## 17. Pseudocode for Machine Learning Team

Input:
    latent vector z
    physical parameters theta
    image size H, W
    time samples t[1:N]

Generate signals:
    Vx = horizontal_signal(t, z)
    Vy = vertical_signal(t, z)
    B  = brightness_signal(t, z)

Compute deflection sensitivities:
    Sx = (Lx / (2 * dx * Va)) * (Dx + Lx / 2)
    Sy = (Ly / (2 * dy * Va)) * (Dy + Ly / 2)

Compute beam trajectory:
    x = Sx * Vx
    y = Sy * Vy

Initialize image:
    I = zeros(H, W)

For each time step n:
    for each nearby pixel (i, j):
        I[i, j] += B[n] * K(i - y[n], j - x[n])

If phosphor persistence is used:
    apply exponential temporal decay

Output:
    generated CRT-like image I

## 18. Equation-to-Implementation Mapping

| Equation | Meaning | Implementation role |
| --- | --- | --- |
| eVa=12mv2 | Electron acceleration | Compute beam speed |
| v=2eVam | Electron velocity | Physical parameter computation |
| E=Vdd | Deflection electric field | Convert voltage to field |
| F=eE | Electron force | Physics consistency |
| a=eVdmd | Transverse acceleration | Beam motion |
| xt=SxVxt | Horizontal position | Beam coordinate |
| yt=SyVyt | Vertical position | Beam coordinate |
| Kx,y | Beam spot kernel | Differentiable image deposition |
| Pt=P0e-t/τ | Phosphor persistence | Temporal glow/decay |
| I=CRTVx,Vy,B,θ | Full generator | ML model function |
| GCRTz→I | Latent generative model | GAN/VAE/diffusion generator |

## 19. Recommended Model Choices

### Option 1: CRT-GAN

Use a neural network to generate:

Vxt,Vyt,Bt,θ

Then pass them through the CRT renderer:

GCRTz=CRTVxt;z,Vyt;z,Bt;z,θ

This is useful when the final output must look realistic and image quality is the main goal.

### Option 2: CRT-VAE

Use an encoder to map a real image into latent space:

qϕz|x

Then decode through the CRT generator:

x=GCRTz

This is useful when interpretability and latent control are important.

### Option 3: CRT-Diffusion

Use diffusion to generate clean brightness or voltage signals from noise, then render them through the CRT model. Diffusion models have demonstrated strong image synthesis performance using iterative denoising (Ho, Jain and Abbeel, 2020).

### Option 4: Physics-Informed CRT Network

Train a neural network while adding physics losses from the CRT equations. This follows the general principle of physics-informed neural networks, where neural models are trained while respecting known physical laws (Raissi, Perdikaris and Karniadakis, 2019).

# References

- Goodfellow, I., Pouget-Abadie, J., Mirza, M., Xu, B., Warde-Farley, D., Ozair, S., Courville, A. and Bengio, Y. (2014) ‘Generative Adversarial Nets’, Advances in Neural Information Processing Systems.

- Ho, J., Jain, A. and Abbeel, P. (2020) ‘Denoising Diffusion Probabilistic Models’, Advances in Neural Information Processing Systems, 33.

- Kingma, D.P. and Welling, M. (2013) ‘Auto-Encoding Variational Bayes’, arXiv:1312.6114.

- Raissi, M., Perdikaris, P. and Karniadakis, G.E. (2019) ‘Physics-informed neural networks: A deep learning framework for solving forward and inverse problems involving nonlinear partial differential equations’, Journal of Computational Physics, 378, pp. 686–707.

- RCA (1966) RCA Receiving Tube Manual RC-25. Radio Corporation of America.

- Frenkel, L. (1960) ‘Electrostatic Deflection Plates for Cathode-Ray Tubes’, Journal of Research of the National Bureau of Standards, 64C(2), pp. 103–111.

- XYZs of Oscilloscopes Primer | Tektronix. (n.d.). Retrieved April 25, 2026, from https://www.tek.com/en/documents/primer/xyzs-oscilloscopes-primer